# moveNode GUI Custom Application

This custom application allows one to modify the location of a picked model node interactively. Controls are provided to translate the node. The ‘Move Node’ dialog box is created by the custom app in the new ‘Edit Mesh’ module and can be used to translate the picked node.

Instructions:

The custom application may be run as follows:

- Extract the contents of the ZIP file moveNodeApp.zip to a new directory.

- Start ABAQUS/CAE using the following command:

> abq643 cae -custom moveNodeApp nostartup

- Open your existing database in ABAQUS/CAE or import a model (.inp file).

- Switch to the new ‘Edit Mesh’ module and select ‘Edit Mesh -> Pick Node’ from the main menu (or click the icon).

- Pick the node to move and use the arrow keys in the ‘Move Node’ dialog to translate it interactively.

- The motion of the node can be specified as a percentage of model dimension or absolute model units.

- The ‘Reset Node to original position’ button can be used as an undo option.